1. Direct Communication (The Common Ground)

Both standards prioritize direct Vehicle-to-Vehicle (V2V) and Vehicle-to-Infrastructure (V2I) communication for safety because it offers the lowest latency and works even if the cellular network is down or congested.

- DSRC: Uses IEEE 802.11p (a Wi-Fi derivative).  It operates in "ad-hoc" mode, meaning cars talk directly to each other like two laptops connecting via Wi-Fi Direct, with no central coordinator.

- C-V2X: Uses the PC5 interface (also called "Sidelink").  This is a specific mode within the cellular standard that allows devices to talk directly to each other over the 5.9 GHz band without passing through a cell tower or requiring a SIM card for authentication in basic safety modes.

2. The Key Difference: Radio Technology

While both are "direct," they manage the radio signals differently:

- DSRC (Wi-Fi based): Uses a "listen-before-talk" method (CSMA/CA). A vehicle listens to the channel; if it's quiet, it transmits. If two cars transmit at once, the messages collide and are lost. This can struggle in very high traffic density.

- C-V2X (Cellular based): Uses a scheduled or sensing-based method (SPS). Even in direct mode, the radio is designed to reserve specific time/frequency slots, making it more robust against collisions in heavy traffic and offering better range/performance in non-line-of-sight scenarios.

3. When C-V2X Uses the Network

C-V2X has a second mode called Uu (the standard cellular link to a tower) which DSRC lacks.

- Direct (PC5): Used for immediate safety (e.g., "I am braking now"). No network needed.

- Network (Uu): Used for long-range data (e.g., traffic light timing ahead, cloud updates). Network needed.

DSRC is only direct; it cannot connect to the cellular core network natively without additional hardware/gateways.